40 acre Center Pivot System Example

Estimating Annual Irrigation Operation Costs 3-10

Standard 40 acre system with end gun, installed at a cost of $38,000 with a well and power source cost of $30,000.

A standard procedure is outlined on page 7 of MSU Extension Bulletin E-2131, “Custom Work Rates in Michigan”. The following would be modification of the DIRTI #5 pricing system.

Ownership Cost

Depreciation: (original cost - salvage value)/years of use

(Pivot cost + installation cost - salvage value)/10 years=

($38,000 - 10,000)/10 = $2,800

(well cost - salvage value)/20 years =

($30,000 - 10,000)/20 = $1,000

Interest:  interest rate * average investment value

6.5% * (original cost + salvage value/2) =

6.5% * ($68,000 + $20,000/2) =

6.5% * $ 44,000 = $ 2,860

Repair:  estimated to between 2 to 5% of original cost

pivot cost * 3% =

$38,000 * 3% = $1,140

well cost * 2% =

$30,000 * 2% = $ 600

Taxes:  no personal property tax in Michigan

the addition irrigation equipment should not increase property taxes

Insurance: estimated at 0.5% * average investment value

0.5% * (original cost + salvage value/2) =

0.5% * ($68,000 + $20,000/2) =

0.5% * $ 44,000   = $220

Total Ownership Cost = $3,800 + $ 2,860 + $1,140 + $220 = $8,020

$8,020 / 33.7 acres = $237.98 /irrigated acre/year

Operating Cost (per acre) or total actual annual cost

These costs are best handled annually calculated using actual costs at the end of season.

Power: use actual fuel or power bill is recommended

estimated power cost:

$3.25/acre in. * 6 in. * 33.7 acres = $657 annually

$3 to $5.50/acre in. range

Labor cost:  recommend use of actual labor bills

Range of $1 to $3.50/acre in.

$1 * 6 acre in. * 33.7 acres = $202 annually

Total Operating cost annually	 = $ 859

$859 /33.7 acres = $25.49/irrigated acre/year

Grand Total Estimated Annual Cost:	$8,020 + $859 = $8,879

$8,879 /33.7 acres = $263.47 /acre at 6"