Single Tower Towable Center Pivot, 5 Circle System Example

Estimating Annual Irrigation Operation Costs 3-10

Single tower towable pivot made up from a 210ft span and a 85ft. covering 8.5 acres on five circles with end gun, installed at a cost of $20,000. Additional tow location at a cost of $1,500 each with a well and power source cost of $18,000, Maybe either gas powered pivot with a 660 ft. hose and reel or 3 phase electric with buried pipe and wire. A standard procedure is outlined on page 7 of MSU Extension Bulletin E-2131, “Custom Work Rates in Michigan”. The following would be modification of the DIRTI #5 pricing system.

Ownership Cost

Depreciation: (original cost - salvage value)/years of use

(Pivot cost + additional anchor points - salvage value)/10 years=

($20,000 + 6 * $1,500) - $7,500 /10 = ($20,000 +$9,000) - $7,500 /10 =$2,150

(well cost - salvage value) / 20 years =

($18,000 - $6,000) / 20 = $600

= $2,750

Interest:  interest rate * average investment value

6.5% * (original cost + salvage value/2) =

Pivot and Install	6.5% * ($29,000 +$7,500 /2) = $1,186

Well      6.5% * ($18,000 +$6,000 /2) = $   780

= $1,966

Repair:  estimated to between 2 to 5% of original cost

Pivot and installation cost * 3% =

$29,000 * 3% = $870

well cost * 2% =

$18,000 * 2% = $ 360

= $1,230

Taxes:  no personal property tax in Michigan

the addition irrigation equipment should not increase property taxes

Insurance: estimated at 0.5% * average investment value

0.5% * (original cost + salvage value/2) =

0.5% * ($20,000 +$9,000 /2) = $73

0.5% * ($18,000 +$6,000 /2) = $60

= $133

Total Ownership Cost = $2,750 + $1,966 + $1,230 + $133 = $6,079

$6,079 / 51 acres = $119.20 /irrigated acre/year

Operating Cost (per acre) or total actual annual cost

These costs are best handled annually calculated using actual costs at the end of season.

Power: use actual fuel or power bill is recommended estimated power cost:

$3.25/acre in. * 6 in. * 51 acres = $995 annually

Range of  $1.80 to $5.50/acre in.

Labor cost:  recommend use of actual labor bills

$1 * 6 acre in. * 51 acres = $306 annually

Range of $1 to $3.50/acre in.

Total Operating cost annually	 = $ 1,301,          $ 1,301 / 51 acres = $25.51 /irrigated acre/year

Grand Total Estimated Annual Cost:

Annual Ownership Cost + Annual Operating cost

$119.20 /irrigated acre/year + $25.51 /irrigated acre/year =  $ 144.71 /irrigated acre/year at 6"