MICHIGAN STATE UNIVERSITY EXTENSION PURDUE EXTENSION

Lyndon Kelley, Extension Educator, Irrigation

612 E. Main Street

Centreville, MI  49032

PHONE  269-467-5511

FAX  269-467-5641

TT  269-467-5501

Six Easy Steps to Calibrating a

Center Pivot for Chemigation

- Calibrate the injector pump.  Determine the injection rate at the injector pump setting to be used.  This must be done with the system running so the injector pump is working against pressure.  To do this, calibrate from the suction side of the injector pump, letting the injector pump draw from a calibration container.  Calibrate by determining the time (in minutes) to pump 1 gallon of liquid.

60

Injector Pump Rate (gal/hr) =

Minutes to Pump 1 Gallon

- Determine the total hours to cover the field at the speed the center pivot will be operated.

Time to Cover Field = Hours

- Determine the total gallons to be injected.  Multiply the injector pump rate (from Step 1) by the total hours to cover the file (from Step 2).

Gallons

Total Gallons to be Injected  = 				         X  (Hours to Cover Field)

Hour

- Determine the chemical amount  required to cover the field.  Multiply the field acreage by the chemical rate as specified.  For nitrogen, it would be pounds N per acre and for pesticides it would be the rate that is recommended on the label.

Chemical Volume

Total Chemical Volume = Field Acres  X

Acre

- Add the chemical (Step 4) to the injection supply tank and then fill the supply tank to the total volume as determined in Step 3.

- Make sure you have a method to agitate the injector supply tank to mix the chemical and keep the chemical in solution (chemicals will settle out if not agitated)

Worksheet for Center Pivot Chemigation Calibration

The following is an example of the basic calculations necessary to determine the injection rate, however, it steps 1 or 6 are not known for sure, then actual field measurements need to be made.

Steps						      Examples

Nitrogen		Pesticide		Your Setup

1.	Determine the number of

acres under center pivot	             125 ac	   	 130 ac		    _____

2.	Amount of chemical per

acre. (28% N: 10#/ac =		   10 gal (30#N)	  2 qts		               _____

3.33 gal)

3.	Multiply steps 1 & 2		    1,250 gal		  260 qts		    _____

4.	If chemicals diluted

with water, total amount			-		       -

of solution.			                  _____                  _____		    _____

5.	Desired water application

rate per acre.				¾”		     .2”			    _____

6.	Determine and set speed of

center pivot for 1 revolution

to apply amount in step 5.	       72 hrs	 	 1,200 min		     _____

7.	Determine injection rate by

dividing step 3 or 4 by step 6.	    17.3 gal/hr 	 .217 qts/min	      _____

8.	Set metering pump and check

delivery rate of the injection

pump to make certain it is

correct.				       36.9 oz/min or   6.94 oz/min or	      _____

1,092 ml/min	  205 ml/min

1 gallon = 128 oz

1 quart = 32 oz

1 pint = 16 oz

1 ounce = 29.6 ml