Estimating Water Usage on Michigan Swine Farms

Jerry May

Michigan State University Extension

Estimating Water Used by Swine Farms in Michigan

Swine farms use well water for watering animals, cleaning facilities, animal cooling and in some instances for moving manure from the barn to the storage structure.  Most pigs are raised in an all-in/all-out environments where one group of pigs, at the same stage of production, is moved into a location and stays there until that group is ready to move to the next location or on to slaughter.  Between groups the facility is thoroughly cleaned by pre-soaking and/or pressure washing.  In the summer, during periods of extreme heat, pigs may be cooled by using drippers which emit small drops of water periodically on the animals back, or by misters giving off a small mist of water intermittently to cool the room.  Some farms use well water to flush manure from the barn to the manure storage structure, but this practice is not very widespread in Michigan and therefore that water was not considered in these calculations.

Table 1 provides the estimated daily water consumption by pigs of various sizes.  The range in daily water consumption within each stage of production is dependant on temperature and water conservation practices on the farm.  For this example the average of the range will be used to estimate daily water use.

Table 1:  Water requirements of pigs1

| Animal type | ### Gal/head/day | ### Animal type | ### Gal/head/day |
| --- | --- | --- | --- |
| Sow and litter | 2.5 - 7 | Finishing pig (100 – 250# BW) | 3 - 5 |
| Nursery pig (up to 60# BW) | .7 | Gestating sow | 3 - 6 |
| Growing pig (60 – 100# BW) | 2 - 3 | Boar | 8 |
|  |  | Replacement Gilt | 3 |

1Swine Care Handbook

Michigan’s average daily and annual water consumption for pigs at various stages of production is provided in Table 2.  The Hog & Pig inventory information is from the 2002-2003 Michigan Agriculture Statistics (NASS).  Hog and pig inventories fluctuate from Quarter to Quarter, therefore the 2002 April 1, June1, September1, and December 1 inventories were averaged to report the 2002 numbers.  The gallon per head per day is the average of the figures provided in Table 1.

|  | Michigan hog and | Gal/hd/ | Gallons | Gallons |
| --- | --- | --- | --- | --- |
| Stage | pig inventory2 | day | daily use | annual use |
| Pigs under 60# | 305,000 | 0.7 | 213,500 | 77,927,500 |
| 60 – 119# | 202,000 | 2.5 | 505,000 | 184,325,000 |
| 120 – 179# | 156,000 | 4 | 624,000 | 227,760,000 |
| Over 180# | 143,000 | 4 | 572,000 | 208,780,000 |
| Gilts | 6,500 | 3 | 19,500 | 7,117,500 |
| Boars | 3,000 | 8 | 24,000 | 8,760,000 |
| Sow and Litter | 12,800 | 5 | 64,000 | 23,360,000 |
| Gestating Sow | 94,200 | 4 | 376,800 | 137,532,000 |
| Total Annual Water for Animal Drinking |  |  | 2,206,650 | 875,562,000 |

Table 2:  Direct water use - Drinking

2NASS

It was more difficult to estimate the indirect water use on swine farms.  There is no reported information on which farms use which practice and the amount of water consumed by each practice.  Table 3 Indirect Water Use – Cleaning and Table 4 Indirect Water Use - Cooling were developed using estimates from individuals working in the field.  It was estimated that about two thirds of the pigs reared in Michigan reside on farms that regularly clean the farms facilities, the remainder of the pigs may reside in pasture or bedded situations where cleaning facilities with water is impractical. Therefore the number of head or litters was multiplied by 67% in determining total water use.

Table 3:  Indirect water use - Cleaning

|  |  |  | % of | Head | Total water |
| --- | --- | --- | --- | --- | --- |
| Operation | Approx. run time | Gal/hr. | pigs | or litters | used |
| Wash Farrowing | 20 hours / 100 litters | 180 | 67% | 184,000 | 4,438,080 |
| Wash Nursery | 4 hrs. / 1000 hd | 180 | 67% | 1,736,000 | 837,446 |
| Wash Finish | 15 hrs. / 1000 hd | 180 | 67% | 1,915,000 | 3,464,235 |
| Pre soak Farr | 1.25 hours / 100 litters | 600 | 67% | 184,000 | 924,600 |
| Pre soak Nursery | .5 hours / 1000 hd | 240 | 67% | 1,736,000 | 139,574 |
| Pre soak Finish | 2 hrs per 1,000 hd | 600 | 67% | 184,000 | 147,936 |
| Total water for cleaning |  |  |  |  | 9,951,872 |

Table  4:  Indirect water use - Cooling

|  | Approx. run time | Days | Gal/hr/ | % of | Head | Total water |
| --- | --- | --- | --- | --- | --- | --- |
| Operation |  | > 80o F | animal | pigs | or litters | used |
| Cool Farrowing | 8 hrs/sow/day | 70 | 2 | 67% | 12,800 | 9,605,120 |
| Cool Breeding | 8 hrs/sow/day | 70 | 2 | 4% | 94,200 | 4,220,160 |
| Cool Finishing | 8 hrs/1,000 hd/day | 70 | 2 | 10% | 501,000 | 56,112 |
| Total water for cooling |  |  |  |  |  | 13,881,392 |

In Michigan larger swine farms may have up 2,500 sows at one location, or up to 4,000 finishing animals at one location.  Michigan has farms that control more animals but the 2,500 sows and 4,000 finishing animals threshold represents the upper ranges of animals at one location being provided water from one water system.  Using the figures in Tables 2, 3, and 4 one may estimate that a location with 2,500 sows will use 4.84 million gallons of water annually (13,262 gallons per day), and a 4,000 head finishing location would use 5.13 million gallons of water annually (14,055 gallons per day).

Because of the swine industry structure, where large farms contract with other farms for growing pigs, it is estimated that there are no swine farms in Michigan that individually consume more than 100,000 gallons of water per day.

In Michigan, the estimated annual water used by swine farms is 899.39 million gallons.  The water used for cooling is at low rates, on hot days when buildings are being well ventilated, therefore all of the cooling water should be considered as evaporative (consumptive).  Very little of the water used for cleaning evaporates, therefore all of the cleaning water should be considered as non-consumptive.

Michigan does not have a large hog processor in the state therefore most of the market hogs produced in the state are shipped out of state for processing.  There is a large cull sow processor in the state and most cull sows stay in Michigan for processing.

Market hogs are approximately 50% water (Tri-State Swine Nutrition Guide).  The 2000 PigChamp Benchmarking publication reports that in year 2000 Michigan’s sow herd had a 47% replacement rate (sows that are sold and replaced with younger gilts) (PigChamp). Michigan Agriculture Statistical Services (NASS) shows that in 2002 Michigan marketed 2.03 million head of hogs with a total weight of 522.9 million pounds.  Using the PigChamp culling rate and the 2002 sow inventory, one may calculate that in 2002 there were 50,000 sows culled weighing approximately 17.60 million pounds and containing 1.1 million gallons water.  Subtracting the cull sow sales from the total 2002 hogs sales indicates that Michigan’s hog producers sold 1.98 million market hogs weighing approximately 505.3 million pounds 2002 (31.58 million gallons water).   Table 5 provides the total consumptive/non-consumptive water use in Michigan.

Table 5:  Consumptive water use

| Water use | Consumptive3 | Non-consumptive3 | Totals3 |
| --- | --- | --- | --- |
| Raising Market Hogs | 31.58 | 667.21 | 698.79 |
| Maintaining Breeding Herd |  | 176.77 | 176.77 |
| Cooling Facilities | 13.88 |  | 13.88 |
| Cleaning Facilities |  | 9.95 | 9.95 |
| Totals | 45.46 | 853.93 | 899.39 |

3 Million Gallons

Bibliography:

National Agriculture Statistic Service (NASS) “Michigan Agriculture Statistics 2002-	2003” available on line at: http://www.nass.usda.gov/mi/stats03/statstext.html

PigCHAMP “Global Benchmarking in Swine Herds” PigCHAMP Inc., 2000

Swine Care Handbook:  National Pork Board, Des Monies, IA, 2002,

Tri-State Swine Nutrition Guide:  Published by: Ohio State University, in cooperation 	with Purdue University Extension, Ohio State University Extension, and 	Michigan State University Extension, 1998